|  |  | # Environmental Health and Safety # STANDARD OPERATING GUIDELINE # Peroxide Formers Test SOP |  |  |
| --- | --- | --- | --- | --- |
| # Section: | # Environmental Health & Safety Management System |  | # Date of Issue: | # 12/22/2017 |
|  |  |  | # Issued By: | # Environmental Health &Safety |
| # Part: | # Health & Safety Guidelines |  | # Revision #: | # 2 |
|  |  |  | # Revision Date: | # 12/21/2017 |
| # Pages: | # 5 |  | # Revised By: | # Judith A. Foster |

Purpose/Scope

  - The purpose of this SOP is to document the Peroxide Test Method for inorganic peroxides in aqueous solutions and organic solvents.

Responsibilities

  - Environmental Health & Safety: Prepare procedure for peroxide tests and regulate the processes according to applicable regulations.

  - Peroxide Users: Execute the procedure when managing chemical peroxides in the store rooms.

References

  - Quantofix® Peroxide 25 – Appendix 1

  - Peroxide Former Labels – Appendix 2

  - RIT Chemical Hygiene Plan

Methodology

  - Peroxidase transfers peroxide oxygen to an organic redox indicator. This produces a blue oxidation product. The peroxide concentration is measured semi-quantitatively by visual comparison of the reaction zone of the test strip with the fields of a color scale.

Applications

  - This test measures inorganic peroxides in aqueous solutions and organic solvents. Polymeric peroxides are not at all or only incompletely measured.

  - This test will be conducted every 3 OR 6 months according to the RITListingofParticularlyHazardousChemical2017final.xls (Found on the EHS Lab Safety webpage).

Measuring Range and Limitations

  - All peroxide chemicals must be disposed after 18 months in inventory or if the peroxide concentration is greater than or equal to 5 mg/l.

  - The test strips can be used for peroxide concentrations up to 25 mg/l. A solution that has a peroxide concentration of more than 5 mg/l must be diluted with distilled water or peroxide-free ether.

  - The test strips must be stored away from sunlight and moisture.  Keep container cool and < 30°C. They are stable up to the expiration date on the box.

  - The pH of the solution must be between 2-12.

Interferences

  - Strong acidic solutions must be buffered with sodium acetate and alkaline solutions with citric acid to bring the pH to a range between 5-7.

Procedure

  - Aqueous Solutions

    - Immerse the reaction zone of the test strip in the sample for 1 second.

    - Allow excess liquid to drip off the end of the test strip.

    - Gently fan the test strip by waving it back and forth in the hood. After about 15 seconds the reaction zone should have changed color according to the peroxide concentration.

    - Read the peroxide concentration from the corresponding color on the scale.

    - Record concentration on the special Peroxide Formers Label associated with the chemical container by marking “Greater than or equal to 5 mg/l” or “Less than 5 mg/l.”

    - If the peroxide concentration is greater than or equal to 5 mg/l, the chemical is to be disposed of as soon as possible.

    - Place used strips in with regular trash.

  - Organic Solvents

    - Immerse the reaction zone of the test strip in the sample for 1 second.

    - Allow excess liquid to drip off the end of the test strip.

    - After the solvent has evaporated, humidify the reaction zone with a drop of distilled water and allow the excess to drip off the strip.

    - Gently fan the test strip by waving it back and forth in the hood. After about 15 seconds the reaction zone should have changed color according to the peroxide concentration.

    - Read the peroxide concentration from the corresponding color on the scale.

    - Record concentration on the special Peroxide Formers Label associated with the chemical container by marking “Greater than or equal to 5 mg/l” or “Less than 5 mg/l.”

    - If the peroxide concentration is greater than or equal to 5 mg/l, the chemical is to be disposed of as soon as possible.

    - Place used strips in with regular trash.

Appendices

  - Appendix 1 – QUANTOFIX Peroxide 25 Test Document (Q:\EMS\EMS SOGs\Health and Safety\Peroxide Formers)

  - Appendix 2 – Peroxide Former Labels

- 3-Month Peroxide Former Label

- 6-Month Peroxide Former Label